Autonomous Visual Target Tracking Drone

Hackathon Submission: Track A (Visual Target Tracking)

Objective

The objective of this project is to design and develop an AI-powered autonomous drone system capable of perceiving its environment through computer vision and making intelligent flight decisions without human intervention. Specifically, this system detects a target (a person) using a webcam, continuously tracks them, maintains a predefined distance, and automatically adjusts its movement to keep the target centered.

Approach Used

This project bridges perception and control by utilizing a lightweight object detection model (YOLOv8n) running in real-time alongside a drone flight controller (PX4 SITL) via MAVSDK.

To ensure safe and reliable flight decisions, the system implements a robust 3D control logic:

- Yaw Control (Horizontal Tracking): The error between the target's bounding box center and the camera's center frame is calculated. Proportional gain is applied to command the drone's yaw rotation speed, allowing it to smoothly pan and keep the target centered.

- Distance Control (Depth Tracking): The area of the target's bounding box is used to estimate depth. If the bounding box is smaller than the target area ($< 80,000$ pixels), the drone pitches forward to catch up. If it is larger, it pitches backward to maintain a safe distance.

- Altitude Hover-Lock: To prevent "ground slam" crashes caused by the target moving lower in the camera frame, the drone's vertical velocity is locked to $0.0$ m/s after takeoff. It acts as an aerial hovercraft, maintaining its 2.5m takeoff altitude while tracking in 2D space.

- Search Mode State Machine: If the target is lost or walks out of frame, the drone automatically engages its safety brakes (halting all momentum) and enters a "Search Sweep" mode, rotating in place at 15 deg/s until the target is re-acquired.

Libraries and Tools

- Python 3.10+: Core application logic.

- Ultralytics YOLOv8: Real-time object detection and bounding box generation.

- OpenCV: Image processing, camera interfacing, and visual debugging.

- MAVSDK-Python: Ground Control Station (GCS) API to send offboard velocity commands.

- PX4 SITL & Gazebo Harmonic: Drone physics simulation and 3D environment rendering.

Setup Instructions

1. Prerequisites

- Windows 10/11 with WSL2 (Ubuntu 22.04) installed.

- Python virtual environment setup on the Windows host.

2. Start the Drone Simulator (Linux/WSL)

Open your Ubuntu terminal and navigate to the PX4-Autopilot directory. To bridge the WSL2 network so the simulator can communicate with Windows, run:

Bash

export PX4_SIM_HOST_ADDR=$(cat /etc/resolv.conf | grep nameserver | awk '{print $2}')

make px4_sitl gz_x500

Wait for the Gazebo 3D simulation window to open and the drone to initialize on the launchpad.

3. Run the Autonomous Tracker (Windows)

- Open your Windows terminal (PowerShell or VS Code), activate your Python virtual environment, and install the required dependencies:

PowerShell

pip install mavsdk ultralytics opencv-python

- Find the internal IP address of your WSL instance:

PowerShell

wsl -d Ubuntu-22.04 hostname -I

- Update line 8 in autonomous_tracker.py with this IP address:

Python

await drone.connect(system_address="udpout://<YOUR_WSL_IP>:14580")

- Run the tracker:

PowerShell

python autonomous_tracker.py

The script will wait for the drone to establish a GPS lock and pass pre-flight checks before automatically arming, taking off to 2.5m, and activating the webcam to begin tracking.

Demonstration Video

https://drive.google.com/file/d/11ein57lz1enEkYAUhtu6pH6mifdShMoF/view?usp=sharing

The video demonstrates the drone taking off autonomously, tracking a moving person, maintaining distance, and executing a search sweep when the target is lost.

GitHub repository:

https://github.com/mdsami275/drone_tracking

Author: Mohammed Samiullah

Institution: Muffakham Jah College of Engineering and Technology, Osmania University